CPE ALL API 명세서

베이스 URL: /api/cpe-all

공통 규칙

- 기본 CRUD: GET(list), POST(create), GET(id), PUT/PATCH(id), DELETE(id)

- 인증: schedule-item만 필요

1) 표준품셈 (Construction Productivity)

엔드포인트

- /productivity/ (CRUD)

- /productivity/project/{project_id}/ (프로젝트별 목록)

POST /productivity/ 요청

{

"project": "UUID",

"main_category": "토공",

"category": "굴착",

"sub_category": "토사",

"item_name": "토사 굴착",

"standard": "표준",

"unit": "m3",

"crew_composition_text": "기능공2/보통인부3",

"productivity_type": "표준품셈",

"skill_worker_1_pum": 1.2,

"skill_worker_1_count": 2,

"skill_worker_2_pum": 0.8,

"skill_worker_2_count": 1,

"special_worker_pum": 0.0,

"special_worker_count": 0.0,

"common_worker_pum": 1.5,

"common_worker_count": 3,

"equipment_pum": 1.0,

"equipment_count": 1,

"pumsam_workload": 100.0,

"molit_workload": 110.0

}

응답

{

"id": 1,

"project": "UUID",

"main_category": "토공",

"category": "굴착",

"sub_category": "토사",

"item_name": "토사 굴착",

"standard": "표준",

"unit": "m3",

"crew_composition_text": "기능공2/보통인부3",

"productivity_type": "표준품셈",

"skill_worker_1_pum": 1.2,

"skill_worker_1_count": 2,

"skill_worker_2_pum": 0.8,

"skill_worker_2_count": 1,

"special_worker_pum": 0.0,

"special_worker_count": 0.0,

"common_worker_pum": 1.5,

"common_worker_count": 3,

"equipment_pum": 1.0,

"equipment_count": 1,

"pumsam_workload": 100.0,

"molit_workload": 110.0

}

2) CIP 생산성 근거/결과/기준표

엔드포인트

- /cip-basis/ (CRUD)

- /cip-result/ (CRUD)

- /cip-standard/ (CRUD)

POST /cip-basis/ 요청

{

"project": "UUID",

"description": "CIP 기본",

"t1": 3.0,

"drill_diameter": 600.0,

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_weathered": 2.0,

"layer_depth_soft_rock": 1.0,

"layer_depth_hard_rock": 0.5,

"layer_depth_mixed": 0.0,

"t3": 2.0,

"concrete_pouring_length": 10.0,

"classification_factor": 0.8,

"total_depth": 11.5,

"t2": 120.0,

"t4": 30.0,

"cycle_time": 155.0,

"daily_production_count": 3.1,

"calculation_formula": "T=t1+t2+t3+t4"

}

응답

{

"id": 1,

"project": "UUID",

"description": "CIP 기본",

"t1": 3.0,

"drill_diameter": 600.0,

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_weathered": 2.0,

"layer_depth_soft_rock": 1.0,

"layer_depth_hard_rock": 0.5,

"layer_depth_mixed": 0.0,

"t3": 2.0,

"concrete_pouring_length": 10.0,

"classification_factor": 0.8,

"total_depth": 11.5,

"t2": 120.0,

"t4": 30.0,

"cycle_time": 155.0,

"daily_production_count": 3.1,

"calculation_formula": "T=t1+t2+t3+t4"

}

POST /cip-result/ 요청

{

"project": "UUID",

"diameter_selection": "600",

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_weathered": 2.0,

"layer_depth_soft_rock": 1.0,

"layer_depth_hard_rock": 0.5,

"layer_depth_mixed": 0.0,

"bit_type_clay": "AUGER",

"bit_type_sand": "AUGER",

"bit_type_weathered": "IMPROVED",

"bit_type_soft_rock": "IMPROVED",

"bit_type_hard_rock": "HAMMER",

"bit_type_mixed": "AUGER",

"total_depth": 11.5,

"t2": 120.0,

"cycle_time": 155.0,

"daily_production_count": 3.1

}

응답

{

"id": 1,

"project": "UUID",

"diameter_selection": "600",

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_weathered": 2.0,

"layer_depth_soft_rock": 1.0,

"layer_depth_hard_rock": 0.5,

"layer_depth_mixed": 0.0,

"bit_type_clay": "AUGER",

"bit_type_sand": "AUGER",

"bit_type_weathered": "IMPROVED",

"bit_type_soft_rock": "IMPROVED",

"bit_type_hard_rock": "HAMMER",

"bit_type_mixed": "AUGER",

"total_depth": 11.5,

"t2": 120.0,

"cycle_time": 155.0,

"daily_production_count": 3.1

}

POST /cip-standard/ 요청

{

"bit_type": "AUGER",

"diameter_spec": "500~600",

"value_clay": 2.1,

"value_sand": 2.3,

"value_weathered": 3.0,

"value_soft_rock": 3.5,

"value_hard_rock": 4.0,

"value_mixed": 2.8

}

응답

{

"id": 1,

"bit_type": "AUGER",

"diameter_spec": "500~600",

"value_clay": 2.1,

"value_sand": 2.3,

"value_weathered": 3.0,

"value_soft_rock": 3.5,

"value_hard_rock": 4.0,

"value_mixed": 2.8

}

3) 기성말뚝 생산성 근거/결과/기준표

엔드포인트

- /pile-basis/ (CRUD)

- /pile-result/ (CRUD)

- /pile-standard/ (CRUD)

POST /pile-basis/ 요청

{

"project": "UUID",

"description": "Pile 기준",

"pile_diameter": 500.0,

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_weathered": 2.0,

"layer_depth_soft_rock": 1.0,

"layer_depth_hard_rock": 0.5,

"layer_depth_mixed": 0.0,

"t1": 5.0,

"t3": 8.0,

"grouting_length": 2.0,

"t5": 18.0,

"welding_diameter": 500.0,

"classification_factor": 0.85,

"total_depth": 11.5,

"t2": 60.0,

"t4": 10.0,

"cycle_time": 96.0,

"daily_production_count": 5.0,

"calculation_formula": "T=t1+t2+t3+t4+t5"

}

응답

{

"id": 1,

"project": "UUID",

"description": "Pile 기준",

"pile_diameter": 500.0,

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_weathered": 2.0,

"layer_depth_soft_rock": 1.0,

"layer_depth_hard_rock": 0.5,

"layer_depth_mixed": 0.0,

"t1": 5.0,

"t3": 8.0,

"grouting_length": 2.0,

"t5": 18.0,

"welding_diameter": 500.0,

"classification_factor": 0.85,

"total_depth": 11.5,

"t2": 60.0,

"t4": 10.0,

"cycle_time": 96.0,

"daily_production_count": 5.0,

"calculation_formula": "T=t1+t2+t3+t4+t5"

}

POST /pile-result/ 요청

{

"project": "UUID",

"diameter_selection": "500",

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_weathered": 2.0,

"layer_depth_soft_rock": 1.0,

"layer_depth_hard_rock": 0.5,

"layer_depth_mixed": 0.0,

"pile_type_clay": "AUGER",

"pile_type_sand": "AUGER",

"pile_type_weathered": "IMPROVED",

"pile_type_soft_rock": "IMPROVED",

"pile_type_hard_rock": "HAMMER",

"pile_type_mixed": "AUGER",

"t1": 5.0,

"t3": 8.0,

"grouting_length": 2.0,

"t5": 18.0,

"welding_diameter": 500.0,

"classification_factor": 0.85,

"total_depth": 11.5,

"t2": 60.0,

"t4": 10.0,

"cycle_time": 96.0,

"daily_production_count": 5.0

}

응답

{

"id": 1,

"project": "UUID",

"diameter_selection": "500",

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_weathered": 2.0,

"layer_depth_soft_rock": 1.0,

"layer_depth_hard_rock": 0.5,

"layer_depth_mixed": 0.0,

"pile_type_clay": "AUGER",

"pile_type_sand": "AUGER",

"pile_type_weathered": "IMPROVED",

"pile_type_soft_rock": "IMPROVED",

"pile_type_hard_rock": "HAMMER",

"pile_type_mixed": "AUGER",

"t1": 5.0,

"t3": 8.0,

"grouting_length": 2.0,

"t5": 18.0,

"welding_diameter": 500.0,

"classification_factor": 0.85,

"total_depth": 11.5,

"t2": 60.0,

"t4": 10.0,

"cycle_time": 96.0,

"daily_production_count": 5.0

}

POST /pile-standard/ 요청

{

"pile_type": "AUGER",

"diameter_spec": "500",

"value_clay": 1.5,

"value_sand": 1.7,

"value_weathered": 2.2,

"value_soft_rock": 2.5,

"value_hard_rock": 3.0,

"value_mixed": 2.0

}

응답

{

"id": 1,

"pile_type": "AUGER",

"diameter_spec": "500",

"value_clay": 1.5,

"value_sand": 1.7,

"value_weathered": 2.2,

"value_soft_rock": 2.5,

"value_hard_rock": 3.0,

"value_mixed": 2.0

}

4) 현장타설말뚝 생산성 근거/결과/기준표

엔드포인트

- /bored-pile-basis/ (CRUD)

- /bored-pile-result/ (CRUD)

- /bored-pile-standard/ (CRUD)

POST /bored-pile-basis/ 요청

{

"project": "UUID",

"description": "Bored Pile 기준",

"t1": 2.0,

"method": "RCD",

"pile_diameter": 1500.0,

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_gravel": 2.0,

"layer_depth_weathered": 1.0,

"layer_depth_soft_rock": 0.5,

"layer_depth_hard_rock": 0.2,

"classification_factor": 0.85,

"total_depth": 11.7,

"t2": 20.0,

"cycle_time": 25.0,

"daily_production_count": 1.6,

"calculation_formula": "T=t1+t2"

}

응답

{

"id": 1,

"project": "UUID",

"description": "Bored Pile 기준",

"t1": 2.0,

"method": "RCD",

"pile_diameter": 1500.0,

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_gravel": 2.0,

"layer_depth_weathered": 1.0,

"layer_depth_soft_rock": 0.5,

"layer_depth_hard_rock": 0.2,

"classification_factor": 0.85,

"total_depth": 11.7,

"t2": 20.0,

"cycle_time": 25.0,

"daily_production_count": 1.6,

"calculation_formula": "T=t1+t2"

}

POST /bored-pile-result/ 요청

{

"project": "UUID",

"method_selection": "RCD",

"diameter_selection": "1500",

"method_clay": "RCD",

"method_sand": "RCD",

"method_gravel": "OSCILLATOR",

"method_weathered": "RCD",

"method_soft_rock": "RCD",

"method_hard_rock": "ALL_CASING",

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_gravel": 2.0,

"layer_depth_weathered": 1.0,

"layer_depth_soft_rock": 0.5,

"layer_depth_hard_rock": 0.2,

"t1": 2.0,

"classification_factor": 0.85,

"total_depth": 11.7,

"t2": 20.0,

"cycle_time": 25.0,

"daily_production_count": 1.6

}

응답

{

"id": 1,

"project": "UUID",

"method_selection": "RCD",

"diameter_selection": "1500",

"method_clay": "RCD",

"method_sand": "RCD",

"method_gravel": "OSCILLATOR",

"method_weathered": "RCD",

"method_soft_rock": "RCD",

"method_hard_rock": "ALL_CASING",

"layer_depth_clay": 5.0,

"layer_depth_sand": 3.0,

"layer_depth_gravel": 2.0,

"layer_depth_weathered": 1.0,

"layer_depth_soft_rock": 0.5,

"layer_depth_hard_rock": 0.2,

"t1": 2.0,

"classification_factor": 0.85,

"total_depth": 11.7,

"t2": 20.0,

"cycle_time": 25.0,

"daily_production_count": 1.6

}

POST /bored-pile-standard/ 요청

{

"method": "RCD",

"diameter_spec": "1500",

"value_clay": 0.8,

"value_sand": 0.9,

"value_gravel": 1.0,

"value_weathered": 1.2,

"value_soft_rock": 1.5,

"value_hard_rock": 1.8

}

응답

{

"id": 1,

"method": "RCD",

"method_display": "R.C.D",

"diameter_spec": "1500",

"value_clay": 0.8,

"value_sand": 0.9,

"value_gravel": 1.0,

"value_weathered": 1.2,

"value_soft_rock": 1.5,

"value_hard_rock": 1.8

}

5) 공정 스케줄 (Schedule Item)

엔드포인트

- /schedule-item/ (CRUD, 인증 필요)

- /schedule-item/initialize_default/ (POST, 인증 필요)

POST /schedule-item/ 요청

{

"project": "UUID",

"data": {

"items": [

{

"id": "earth-1",

"main_category": "토공사",

"process": "토사운반",

"work_type": "토사운반",

"quantity": 1000.0,

"quantity_formula": "100*10",

"unit": "m3",

"productivity": 50.0,

"crew_size": 5,

"daily_production": 250.0,

"working_days": 4.0,

"operating_rate_type": "EARTH",

"operating_rate_value": 0.75,

"calendar_days": 5.3,

"calendar_months": 0.18,

"front_parallel_days": 0.0,

"back_parallel_days": 0.0,

"remarks": "",

"standard_code": "01-01-001",

"total_workload": 1000.0,

"_startDay": null,

"_parallelGroup": null

}

],

"links": [

{ "from": "earth-1", "to": "earth-2", "type": "FS" }

]

}

}

응답

{

"id": 1,

"project": "UUID",

"data": {

"items": [

{

"id": "earth-1",

"main_category": "토공사",

"process": "토사운반",

"work_type": "토사운반",

"quantity": 1000.0,

"quantity_formula": "100*10",

"unit": "m3",

"productivity": 50.0,

"crew_size": 5,

"daily_production": 250.0,

"working_days": 4.0,

"operating_rate_type": "EARTH",

"operating_rate_value": 0.75,

"calendar_days": 5.3,

"calendar_months": 0.18,

"front_parallel_days": 0.0,

"back_parallel_days": 0.0,

"remarks": "",

"standard_code": "01-01-001",

"total_workload": 1000.0,

"_startDay": null,

"_parallelGroup": null

}

],

"links": [

{ "from": "earth-1", "to": "earth-2", "type": "FS" }

]

}

}

POST /schedule-item/initialize_default/ 요청

{

"project_id": "UUID"

}

응답

{

"message": "Initialized default items"

}